1. Mechanical Basics

1 Pipe and Tube

| Aspect | Pipe | Tube |
| --- | --- | --- |
| Primary Purpose | Transporting fluids or gases | Structural applications and precision work |
| Sizing | Based on nominal diameter (NPS) | Based on actual outside diameter (OD) |
| Wall Thickness | Specified by schedule (e.g., SCH 40) | Specified by gauge or direct thickness |
| Tolerance | Looser tolerances | Tighter tolerances for OD, ID, wall thickness |
| Shape | Mostly round | Round, square, rectangular, oval, etc. |
| Standardization | ANSI/ASME B36.10M, B36.19M | ASTM A269, A554, etc. |

Using Pipe: Transporting fluids/gases.

Transporting fluids, steam, oil, gas, water, etc.

Pressure and flow rate are critical.

You are working with plumbing, process piping, or HVAC systems.

Using Tube: Structural, precision, or aesthetic needs.

Structural applications or need tight dimensional control.

Aesthetic appearance or precision is important (e.g., handrails, medical devices).

You need custom shapes or lightweight materials.

Description both of them in BOM:

SS Pipe 3” Sch. 40

SS Sanitary Tube 3”      (Sanitary is only based on requirement)

(The Details Showing in Table 1 refer to Appendix)

2 Sheet Metal, Plate, Flat Bar

| Feature | Sheet Metal | Plate | Flat Bar |
| --- | --- | --- | --- |
| Definition | Thin, flat rolled metal | Thick, flat rolled metal | Long, rectangular section with set width and thickness |
| Thickness | < 3 mm (typically) | ≥ 3 mm (varies by standard, often 6+ mm) | Varies, often 3–25 mm; defined cross-section |
| Form | Supplied in sheets or coils | Supplied in flat sheets or plates | Supplied as bars (long strips) |
| Common Shapes | Rectangular sheets or coils | Rectangular sheets | Rectangular cross-section (e.g., 25×3 mm) |
| Use Case | Ductwork, panels, enclosures | Heavy-duty parts, structural components | Brackets, stiffeners, supports |
| Processing | Often cut, bent, or stamped | Machined, drilled, welded | Cut, drilled, welded |

Description of them in BOM:

SS SHT 12 Gauges(Ga) (2B)         (2B is surface treatment)

SS Plate 3/8” x 7” x 10”                 (Here is thickness x width x length, in general, SS Plate 3/8”)

SS Flat Bar (F.B.) 7” x 3” x 10”     (Here is width x thickness x length)

3 Bolt and Screw

| Term | Description | Typical Use Case |
| --- | --- | --- |
| Screw | A fastener with external threads that engages directly into a material (e.g., wood, plastic, metal). Often forms its own mating thread. | Fastening into tapped holes or soft material |
| Bolt | A fastener with external threads designed to be used with a nut (or into a pre-tapped hole). Typically, not self-tapping. | Clamping parts together with nut & washer |
| Thread | The helical ridge on a screw/bolt or inside a nut/hole. Can be external (male) or internal (female). | Allows mating between components |
| Stud (Ready Rod) | A rod threaded on both ends (or sometimes fully). One end screws into a part; the other receives a nut. No head. | Engine blocks, heavy equipment, flanged joints |

Description of them in BOM:

2 x M3 x 1.25-6H/7g-LH 5   (Completely Penetrate)

¼ NPT5”  ┕┙2

3/8 NPS5”   Hole7”

| Abbreviation | Full Name | Thread Type | Application | Tapered | Units |
| --- | --- | --- | --- | --- | --- |
| NPT | National Pipe Taper | Pipe thread (tapered) | Sealing for fluid/gas (tight fit) | ✅ | Inch |
| NPS | National Pipe Straight | Pipe thread (straight) | Fluid/gas with gasket/seal | ❌ | Inch |
| UNC (NC) | Unified National Coarse | Fastener thread | General bolts/screws (coarse pitch) | ❌ | Inch |
| UNF (NF) | Unified National Fine | Fastener thread | Precision bolts/screws (fine pitch) | ❌ | Inch |
| UNEF | Unified National Extra Fine | Fastener thread | High precision/light duty | ❌ | Inch |
| M | Metric Thread | Fastener thread | International bolts/screws | ❌ | Metric |
| BSPT | British Standard Pipe Taper | Pipe thread (tapered) | British/international pipe sealing | ✅ | Inch (British) |
| BSPP | British Standard Pipe Parallel | Pipe thread (straight) | British fittings with gasket | ❌ | Inch (British) |

1/2"-14 NPT → Tapered pipe thread, 1/2 inch nominal size, 14 threads per inch

3/4"-10 UNC → Coarse threaded bolt, 3/4 inch diameter, 10 TPI

M8 × 1.25 → Metric bolt, 8 mm diameter, 1.25 mm pitch

1-NPS → Straight pipe thread, 1 inch nominal size

板子的厚度 = 1~1.5螺纹孔的直径

Le≈1.5~2d  Le​ 是有效螺纹长度, 啮合深度 d为螺纹公称直径

4 Form & Location Tolerance

① Form Tolerance

② Location Tolerance

5 Pump & Motor

5.1 General Type of Pump

| Pump Type | Drive Method | Description |
| --- | --- | --- |
| Centrifugal Pump | Impeller directly driven by motor shaft | Most common pump type, fill water inside |
| Self-Priming Pump | Motor drives impeller + vacuum chamber | Can draw water even when initially dry |
| Piston / Gear Pump | Motor drives piston or gear mechanism | Used for high-pressure or special fluids |
| Submersible Pump | Pump and motor integrated, submerged | Motor operates underwater , used in wells, ponds |
| DC Micro Pump | Motor and pump molded as one (12V/24V) | Commonly used in small projects or models |

Pump needs motor to drive it.

5.2 Calculating the Motor’s Power

- Total Head (pump height)

- Flow Rate (e.g. L/min)

- Power Suppl Type (e.g. 220V AC)

- Application Scenario (Irrigation, drawing water from a well, etc.)

P= ρghQη   (h=Hactual+Hloss;   Hloss=0.3~0.5∙Hactual)

5.3 How to Choose the Type of Pump

| Step |  | Description | Example |
| --- | --- | --- | --- |
| 1 | Define the Application | Pumping, irrigation, water supply, cooling, sewage, circulation? | Example: Irrigation for an orchard |
| 2 | Determine Basic Parameters | - Head H (meters) - Flow Rate Q (L/min or m³/h) - Fluid Type | Need to pump water up to 8 meters, 3 m³/h flow, clean water |
| 3 | Choose Installation Type | - Surface pump / submersible pump / horizontal pump - Self-priming required? | Surface self-priming pump, drawing from a pond |
| 4 | Power Supply | 220V single-phase? 380V three-phase? 12V/24V DC? | 220V household AC available |
| 5 | Match Pump Model | Select model using manufacturer’s catalog (or estimate using empirical formulas) | Match model: JET100, ISG, QB series, etc. |

| Scenario | Recommended Pump Type |
| --- | --- |
| Need to draw water from a low source | Self-priming pump (e.g., JET, WZB series) |
| Need to pump water far and high | Multistage centrifugal pump |
| Pump needs to work underwater | Submersible pump (e.g., QDX, QY series) |
| Water contains particles or sludge | Sewage pump / slurry pump |
| Small-scale irrigation or dispensers | Mini DC pump (12V / 24V) |

The motor choosing is based on the Pump type.

6 Gear, Sprocket, & Bear

Choosing the Sprocket is based on the allowed torque, and transmission ratio.

The torque of sprocket is:

P=T∙ω,  ω=2πn60 ≈ (n is rmp)

Treal=Ks∙T  (Ks is the service factor)

Treal≤Tmax⁡_allow

小链轮推荐齿数：

工业场合推荐：≥ 17 齿

尽量不要 < 13 齿，避免弯曲疲劳过大

7 Plastic Material

Common Plastic Materials – Properties, Applications & Design Notes

| Material | Key Properties | Typical Design Applications | Design Considerations |
| --- | --- | --- | --- |
| ABS (Acrylonitrile Butadiene Styrene) | Good moldability, good surface finish, moderate rigidity | Toys, appliance housings, handles | Poor UV resistance, prone to aging |
| PP (Polypropylene) | Lightweight, chemical-resistant, low cost, live hinge capable | Food containers, hinge structures (e.g. stationery lids) | High shrinkage, difficult to paint |
| PC (Polycarbonate) | High strength, transparent, impact-resistant, heat-resistant | Protective covers, light housings, transparent parts | Expensive, prone to stress cracking during processing |
| PA (Polyamide / Nylon) | High toughness, wear-resistant, heat-resistant, hygroscopic | Gears, sliders, industrial components | Moisture absorption causes dimensional changes |
| PMMA (Acrylic) | High transparency, hard and brittle, scratch-prone | Display cases, lightboxes, signage | Brittle, poor impact resistance |
| POM (Polyoxymethylene / Acetal) | High strength, low friction, dimensional stability | Bearings, slides, mechanical parts | Requires careful mold release design |
| TPU (Thermoplastic Polyurethane) | Excellent elasticity, abrasion resistance, anti-slip, flexible & dyeable | Phone cases, shoe soles, sports accessories | Slightly expensive, narrow processing window |
| PETG (Glycol-modified PET) | Impact-resistant transparent plastic, between PC and PMMA | Transparent containers, medical parts, 3D printing | Tends to string, poor resistance to strong acids |
| PE (HDPE/LDPE) | Chemically resistant, low friction, tough | Bottle caps, piping, children's toys | Poor adhesion, difficult to paint |

Choosing Plastic Material From Various Aspect

| Category |  | Description | Recommended Materials |
| --- | --- | --- | --- |
| 1 | Mechanical Properties | Need for high strength or toughness | PC (impact resistant), PA (high strength), POM (rigid) |
| 2 | Transparency | Need to see internal structures | PMMA, PC (high transparency) |
| 3 | Surface Finish | Need for plating, painting, or etching | ABS (excellent for plating), PC+ABS (great for painting) |
| 4 | Heat Resistance | Must withstand temperatures above 80°C | PC, PPS, PEI |
| 5 | Weather Resistance | Outdoor use requiring aging resistance | ASA, modified PP, UV-stabilized PC |
| 6 | Flame Retardancy | UL94 flame resistance required | Flame-retardant ABS, FR-PC, PA66 FR |
| 7 | Processing Method | Type of process: injection, blow, extrusion | ABS/PP (injection molding), PE (blow molding) |
| 8 | Environmental & Food Safety | Requires ROHS/REACH/FDA compliance | PP, HDPE, TPU |

| Application | Recommended Materials | Reasons |
| --- | --- | --- |
| Toys / Figures | ABS, PVC, PU | Easy to mold, good surface finish options |
| Medical Device Housings | PC, ABS, PC+ABS | Impact resistant, good formability |
| Transparent Covers / Lamps | PC, PMMA | High transparency, UV resistant |
| Industrial Parts (Gears/Fasteners) | POM, PA66 | High rigidity, wear resistant |
| Daily-use Containers (Food Grade) | PP, PE | Non-toxic, acid/alkali resistant |
| Electronics Housings | PC+ABS, ASA | Impact resistant, easy to process |

The Method of Figure Crafting

| Material Name | Key Properties | Typical Use |
| --- | --- | --- |
| PU Cast Resin (Polyurethane Resin) | Most used for casting, high flowability, fast curing, excellent detail replication | Small-batch casted figure parts |
| PVC (Polyvinyl Chloride) | Thermoplastic, moldable under heat and pressure, good hand feel, low cost | Mass-produced commercial figures (most finished figures) |
| ABS (Acrylonitrile Butadiene Styrene) | Good rigidity, impact resistance, suitable for structural parts | Joints, movable mechanisms, weapon accessories |
| POM (Polyoxymethylene / Acetal) | Ultra-low friction, high wear resistance, mechanically robust | Joint connectors (e.g. articulated action figures) |
| Silicone (for molds) | Soft, excellent mold release, heat-resistant, high-detail replication | Mold-Making material (not for final parts!) |

① Small-Batch Figure Production → PU Resin + RTV Silicone Mold

Step 1: Create the Master Prototype

Use sculpting clay (e.g., oil-based clay or epoxy putty) or print the master model using a high-detail SLA 3D printer.

Step 2: Make the Silicone Mold

Encapsulate the master with two-part RTV silicone (room-temperature vulcanizing silicone).

Vacuum degas to remove air bubbles. Once cured, cut open the mold and remove the master.

Step 3: Resin Casting (Pouring PU Resin)

Mix PU resin parts A and B (e.g., Smooth-On series) and pour into the mold.

The resin cures quickly (within 3 to 10 minutes).

Demold → Trim → Sand the part.

Step 4: Painting and Finishing

First, spray a primer (gray primer like Mr. Surfacer is recommended).

Then paint using model paints (e.g., Mr. Hobby, Tamiya).

Finally, apply a topcoat for protection.

② Mass Production of Commercial Figures → PVC Injection Molding + Metal Molds (Standard Process Used by GSC, ALTER, Kotobukiya, etc.)

Step 1: Create a Digital Model (e.g., using ZBrush, Blender)
3D modeling and digital sculpting → Export slicer-ready files.

Step 2: CNC Machining of Steel Mold
High-precision steel molds are made using CNC milling and electrical discharge machining (EDM).
Mold production is expensive (ranging from tens of thousands to hundreds of thousands of RMB).

Step 3: PVC Heat Injection Molding
PVC pellets (with added plasticizers for softness) are preheated and injected into the mold under high pressure. After cooling, the parts are demolded → Basic shaping is complete.

Step 4: Flash Removal & Assembly
Manual trimming, assembling parts, drilling holes — may include internal metal rods or supports.

Step 5: Painting (Mostly Spray + Pad Printing)
Mostly done via automated spray painting, masking templates, and hand-painted detail work.

The Comparison of PU & PVC for Figure Production

| Aspect | PU Resin | PVC (Industrial Mass Production) |
| --- | --- | --- |
| Stage | Prototype making + small-batch casting | Mass production (e.g., GSC figures) |
| Mold Type | RTV silicone mold (flexible) | Hard metal mold (expensive) |
| Processing Method | Manual pouring, room temperature curing | High-temperature injection molding or heat press casting |
| Detail Quality | Excellent — ideal for sculpting prototypes or high-precision, low-volume items | Also, excellent — especially for large, complex parts |
| Disadvantages | High material cost, low efficiency, prone to aging/yellowing | Very expensive molds, high equipment requirements, unfriendly DIY |

8 Metal Material

1. General Metal Material Properties

| Category | Common Metals |  | Key Properties | Application Fields |
| --- | --- | --- | --- | --- |
| Carbon Steel | Q235, 45# Steel | High strength, low cost, weldable, prone to rust |  | Structural components, welded structures, shafts |
| Alloy Steel | 40Cr, 20CrMo | High strength, good after heat treatment, wear-resistant |  | Gears, bearings, connecting rods, heavy-duty parts |
| Stainless Steel | 304, 316 | Corrosion-resistant, aesthetic, expensive, hard to machine |  | Food equipment, medical devices, decorative casings |
| Aluminum Alloy | 6061, 7075 | Lightweight, good thermal conductivity, corrosion-resistant, easy to machine |  | Aerospace parts, structural components, laptop housings |
| Copper Alloy | Brass, Copper (Red) | Excellent electrical and thermal conductivity, corrosion-resistant, expensive |  | Electrical connectors, heat exchangers, plumbing fittings |
| Magnesium Alloy | AZ91D | Ultra-lightweight, good damping, oxidizes easily, recyclable |  | Portable devices, aerospace, 3C electronics |
| Titanium Alloy | TC4 (Ti6Al4V) | High strength, corrosion-resistant, biocompatible, very expensive |  | Aerospace, medical implants, precision machinery |
| Zinc Alloy | Zamak 3, etc. | Easy to die-cast, low cost, good decorative finish |  | Toys, decorative parts, hardware components |

2. Main Performance Indication

| Performance Index |  | Meaning & Applications |
| --- | --- | --- |
| 1 | Tensile Strength | Ability to withstand pulling force; used to evaluate structural strength requirements |
| 2 | Yield Strength | The limit between elastic and plastic deformation; determines if a material will “spring back” |
| 3 | Hardness | Resistance to indentation or scratching; indicates wear resistance |
| 4 | Ductility / Plasticity | Tendency to stretch without breaking; important for formed or stamped parts |
| 5 | Density | Mass-to-volume ratio; crucial for lightweight design |
| 6 | Thermal Conductivity | Rate of heat transfer; affects heat sinks and cookware materials |
| 7 | Electrical Conductivity | Ability to conduct electricity; critical for electronics and wiring |
| 8 | Corrosion Resistance | Resistance to rust or chemical attack; essential for outdoor or humid environments |
| 9 | Weldability | Ease of joining by welding |
| 10 | Machinability | Suitability for milling, turning, stamping, forging, casting, etc. |

Application Recommends

| Application Scenario |  | Recommended Metal Materials | Brief Reason |
| --- | --- | --- | --- |
| 1 | Toy Structures | Aluminum Alloy, Zinc Alloy | Lightweight, easy to process, safe |
| 2 | Mechanical Components | Alloy Steel, Carbon Steel | High strength, good machinability |
| 3 | Corrosive Environments (e.g. seaside) | Stainless Steel 316, Titanium Alloy | Excellent corrosion resistance |
| 4 | Laptop Housings | Aluminum Alloy, Magnesium Alloy | Lightweight, aesthetic, easy for CNC |
| 5 | Electrical Connectors | Copper, Copper Alloys | Excellent conductivity, corrosion-resistant |
| 6 | Precision Aerospace Parts | Titanium Alloy, 7075 Aluminum | High strength, lightweight |

Tips: Material Selection Advice for Designers

- Structure + Cost Priority: Use carbon steel or alloy steel.

- Aesthetic + Corrosion Resistance: Choose stainless steel or anodized aluminum.

- High Thermal/Electrical Conductivity: Copper is the top choice.

- Lightweight + High Formability: Go for aluminum, magnesium, or zinc alloys.

- Decorative Parts with Complex Surface Details: Use zinc alloy die-casting.

- High-End / Medical-Grade Requirements: Option for titanium alloy.